I tryed to Connect to an FTP which runs on an B&R Programming Logical Controller (PLC).

(1) If i used the Anonymous User to log in, FileZilla successfuly connects to the C: Partition on the CF Installed in the PLC. But in this mode i can't change to the F: Partition.

Short-Log:

Befehl:   CWD F:  -> Antwort:	550 Guest access denied.

(2) So i tryed to use an Username (User: 1) with the Password (Pass: 1). This results in an Error after successfuly requesting the current directory.

Short-Log:

Antwort:	257 Current directory is "C:"

Fehler:	Verzeichnisinhalt konnte nicht empfangen werden

(3) If i use WS_FTP the whole thing works, so i thought the Log-File could give you interessting informations ... so i also added it.  Connecting with User:1 and Pass:1

Would be happy if i can work with FileZilla in the Future, because of its other functions.

(1)

Status:	Verbinde mit 10.49.20.118:21...

Status:	Verbindung hergestellt, warte auf Willkommensnachricht...

Antwort:	220 VxWorks FTP server (VxWorks 5.4.2) ready.

Befehl:	USER anonymous

Antwort:	331 Guest login ok, send your complete e-mail address as password.

Befehl:	PASS **************

Antwort:	230 Guest login ok, upload directory is /.

Status:	Verbunden

Status:	Empfange Verzeichnisinhalt...

Befehl:	PWD

Antwort:	257 Current directory is "/"

Befehl:	TYPE I

Antwort:	200 Type set to I, binary mode

Befehl:	PASV

Antwort:	227 Entering Passive Mode (10,49,20,118,4,25)

Befehl:	LIST

Antwort:	150 Opening BINARY mode data connection

Antwort:	226 Transfer complete

Status:	Anzeigen des Verzeichnisinhalts abgeschlossen

(2)

Status:	Verbindung zum Server getrennt

Status:	Verbinde mit 10.49.20.118:21...

Status:	Verbindung hergestellt, warte auf Willkommensnachricht...

Antwort:	220 VxWorks FTP server (VxWorks 5.4.2) ready.

Befehl:	USER 1

Antwort:	331 Password required

Befehl:	PASS *

Antwort:	230 User logged in

Status:	Verbunden

Status:	Empfange Verzeichnisinhalt...

Befehl:	PWD

Antwort:	257 Current directory is "C:"

Fehler:	Verzeichnisinhalt konnte nicht empfangen werden

(3)

Pass = 1

WINSOCK.DLL: WinSock 2.0

WS_FTP LE 5.08 2000.01.13, Copyright © 1992-2000 Ipswitch, Inc.

- -

connecting to 10.49.20.118:21

Connected to 10.49.20.118 port 21

220 VxWorks FTP server (VxWorks 5.4.2) ready.

USER 1

331 Password required

PASS (hidden)

230 User logged in

PWD

257 Current directory is "C:"

SYST

215 UNIX Type: L8 Version: VxWorks

Host type (S): UNIX (standard)

PASV

227 Entering Passive Mode (10,49,20,118,4,29)

connecting to 10.49.20.118:1053

- -

connecting to 10.49.20.118:1053

Connected to 10.49.20.118 port 1053

LIST

150 Opening BINARY mode data connection

Received 310 bytes in 0.1 secs, (30.00 Kbps), transfer succeeded

226 Transfer complete

View C occurs ……  after that i changed the Directory ….

CWD F:

250 Changed directory to "F:"

PWD

257 Current directory is "F:"

PASV

227 Entering Passive Mode (10,49,20,118,4,30)

connecting to 10.49.20.118:1054

- -

connecting to 10.49.20.118:1054

Connected to 10.49.20.118 port 1054

LIST

150 Opening BINARY mode data connection

Received 65 bytes in 0.1 secs, (6500.00 bps), transfer succeeded

226 Transfer complete

View of Path F: Occured

View of Path C:

View of Path F: